CMPE 352  –  Signal Processing & Algorithms
Week 12 Problem Session
Week of 29 April 2019

Problem 1

The compact disc (CD) is an application of PCM. This is a high fidelity situation targeting an audio signal bandwidth of 20 kHz. The actual sampling rate is 44.1 kHz. The signal is quantized into a rather large number of quantization levels L=65'536.

(a) How can you explain or justify the value of 44.1 kHz used for sampling?
(b) How many bits are used to represent each sample? 
(c) What is the bit rate of the PCM stream for the CD?

Problem 2

Let the signal xt be given as xt=e-atut,    (a>0). We define the effective bandwidth of the signal as the frequency interval that contains 90% of the total signal energy. Determine a proper sampling frequency fS to digitize this signal assuming that the signal components  outside of the effective bandwidth can be neglected. (Assume a=1000.)

Note: In the frequency domain, the signal energy is given by the integral -∞∞Xjω2dω.

Problem 3

(a) Determine a digital integration algorithm, along the same lines as the differentiation algorithm explained during the lecture hours.

(b) Draw a signal block-diagram of the algorithm.

(c) Implement the algorithm in Matlab.

(d) Assume that the input is a sinusoidal signal and draw on the same plot the input and output signals for this algorithm.

Problem 4

A signal within the range of (-10,+10)  [V] has an average power of 10 [V2]. With at least how how many bits should this signal be quantized to guarantee a signal-to-quantization-error ratio of 30 dB?